PARIVAR SATHI / GHAR SAJAG

# Ghar Sajag Engineer Handover Guide

How to understand, build, validate and safely extend the current Parivar Sathi software baseline

| Document version | 2.1 |
| --- | --- |
| Reference source | ParivarSathi_v1.5.4_PWA_BatteryAnalytics_v3_4_2 |
| Reference Git state | main / commit 0cc5ca6 (reported after push) |
| Date | 12 September 2026 |
| Status | v2.1 handover guide for validated v3.4.2 host/reference baseline |
| Audience | Firmware, backend, PWA, verification and integration engineers |

Boundary: this document describes the currently validated hardware-independent software. Real ESP32 GPIO/RF, power/charging, flash durability, signed OTA, provider delivery and installed-home acceptance remain separate physical/HIL gates.

# 1. Start here

The current official software baseline is the native-WSL source tree for ParivarSathi_v1.5.4_PWA_BatteryAnalytics_v3_4_2. It is a hardware-independent reference and connected simulator/PWA, not yet flashable commercial ESP-IDF firmware. The most important engineering rule is to preserve deterministic monitoring semantics while replacing host adapters with real hardware/provider adapters.

| Question | Answer |
| --- | --- |
| Where is authoritative source? | Git repository main branch; validated code folder ParivarSathi_v1.5.4_PWA_BatteryAnalytics_v3_4_2. |
| What is the release command? | make release-gate-final |
| What must pass? | Host release gate + mandatory real Chromium PWA gate. Current baseline: PASS + 22/22 Playwright. |
| What is not certified? | Physical ESP32 GPIO/RF/power/flash/OTA/provider and installed-home operation. |
| What is the canonical scenario source? | tests/functional/scenario_catalog.json (92 cases). |
| What should not be committed? | build/, logs/, evidence/, node_modules/, test-results/, playwright-report/, __pycache__, *.pyc. |

# 2. Repository reading order

1.  README.md — current product/release overview and main commands.

2.  IMPLEMENTATION_STATUS.md — current host/software vs physical acceptance boundary.

3.  docs/PRODUCT_ARCHITECTURE_v1.4.md and this architecture/LLD set — module/dependency design.

4.  shared/include/gs/domain.hpp, protocol.hpp, rules.hpp + shared/src/rules.cpp — canonical domain and deterministic rule semantics.

5.  firmware/node and firmware/hub components — hardware-independent node/hub composition.

6.  backend/ghar_sajag — service boundaries, read models, config and battery analytics.

7.  tools/sim/local_lab.py + tools/sim/pwa — connected host lab and product PWA.

8.  tests/functional/scenario_catalog.json + tests/playwright/pwa_release_gate.spec.ts — canonical regression and mandatory browser acceptance.

9.  docs/BATTERY_ANALYTICS_DESIGN.md + tests/python/test_battery_analytics.py — battery model and limits.

10.  tools/hil — future physical provisioning/power/installation harness.

Figure H-1. Mental model for source ownership.

# 3. Build and environment

Official local qualification path — Use ~/projects/Ghar_sajag/... in native WSL. On the current low-memory laptop, /mnt/f is materially slower and showed browser timing flakiness; keep F: for backups/docs/releases, not qualification.

cd ~/projects/Ghar_sajag/code/ParivarSathi_v1.5.4_PWA_BatteryAnalytics_v3_4_2

# first clone / after deleting node_modules
npm install

# complete release qualification
make release-gate-final

If this is a completely new machine, use make playwright-install once to install the project dependency and Chromium/system prerequisites. node_modules is generated local state and remains ignored by Git.

# 4. Current release evidence

Figure H-2. Release evidence stack.

| Layer | Current baseline |
| --- | --- |
| Validation coverage | PASS |
| Contracts | PASS |
| C++ | 94 checks PASS |
| Python | 31 PASS |
| JavaScript | 11 PASS |
| Feature variants | 4 variants PASS |
| Product profiles | Base + AI PASS |
| Dummy streams | 5/5 PASS |
| Functional catalog | 92/92 PASS |
| HTTP | 16/16 PASS |
| Sanitizers/trace | PASS |
| Legacy browser-e2e | PASS in latest full run |
| Mandatory PWA Playwright | 22/22 PASS, desktop + mobile |
| Final release target | PASS |

# 5. Current product behavior a new engineer must understand

## 5.1 Care vs device health

The top banner represents household-care state only.

Specific concerns belong in routine cards; banner wording remains generic.

Low battery, high drain or device operational health never independently turns the care banner red.

The same authorized dashboard can be used by family/caregiver roles; authorization controls actions, not a separate P0 page.

## 5.2 Morning routine

Bedroom starts sequence; bathroom + kitchen after it complete in either order within configured window.

Before-deadline incomplete state is “pending,” not “missing.”

Missing-morning concern requires eligibility (HOME/coverage/trusted time) and turns the generic banner red.

Current v3.4.2 code restarts sequence on another bedroom motion. Do not change this silently; first define desired behavior + add a regression case.

## 5.3 Night, door and inactivity

Night bathroom/common visits use configurable thresholds and a configurable PIR merge interval.

Count equal to threshold is normal; count greater than threshold raises concern.

Door-left-open and post-door inactivity are configurable and shown in Main Door card.

Expected vs unexpected event colors/labels are part of browser regression.

## 5.4 Device Schedules

Family-specific values must be configurable; defaults only seed new homes.

The editor is backend-versioned and must remain open despite periodic polling.

Transport reliability parameters remain engineering-controlled.

## 5.5 Battery analytics

Figure H-3. Battery analytics mental model.

Voltage + cumulative usage counters feed the estimator; current values come from a per-device calibration profile.

The algorithm outputs percent, mAh/day, remaining days, confidence, drain state, wakes/day and RF retries/day.

Current host profile values are synthetic. Hardware calibration is required before trusting field lifetime numbers.

# 6. Change workflow

1.  Define the observable product behavior and the owner of truth (node/hub/backend/PWA).

2.  Change deterministic rules/contracts before UI duplication; keep hardware/provider specifics behind adapters.

3.  Add positive + negative + boundary/recovery coverage in the canonical catalog or lower-level suite.

4.  Run focused tests during development, then make release-gate-final before release candidate.

5.  Inspect Git diff and stage only intended source/docs. Generated evidence and dependency folders stay ignored.

6.  Commit/push only after the tested tree is cleanly understood. If a physical claim is not tested on hardware, keep it marked pending in docs.

# 7. Debugging map

| Symptom | Start here |
| --- | --- |
| Wrong care banner | backend read model care.alert → tools/sim/pwa/app.js banner selector → rule/incident event that set care state. |
| Routine card wrong | RulesCore state + scenario snapshot → backend queries/read model → PWA card presenter. |
| Settings not saved | PWA form serialization → /pwa/action/config path → B06 desired config → state refresh/config_version. |
| Settings editor closes | Check scheduleEditorOpen and render polling path; regression test waits >3.5 s. |
| Battery estimate odd | Check telemetry validity, profile, sample history, counter reset, observation confidence and recent/baseline rates. |
| Care banner turns red on battery | Bug: device-health isolation invariant violated; see Playwright low-battery/high-drain tests. |
| Event ordering confusing | Check actual detection timestamp and newest-first sort in read model/PWA. |
| Playwright module missing | npm install (or make playwright-install on a new machine); node_modules is intentionally not committed. |
| Browser gate flaky on /mnt/f | Move qualification run to native WSL path; do not weaken useful assertions to compensate for filesystem slowness. |

# 8. Physical integration plan

| Gate | Required evidence before pilot |
| --- | --- |
| GPIO/sensors | PIR/reed/button debounce, edge behavior, false triggers, mounting and capability mapping on exact boards. |
| RF | ESP-NOW/BLE/Wi-Fi security, RSSI/range, loss/retry, coexistence, channel/router changes and queue occupancy. |
| Power | Measured deep-sleep/awake/sensor/TX/RX currents, ADC/fuel-gauge, usable cell capacity, charger/UPS transition, brownout. |
| Storage/recovery | Flash commit markers/CRC, wear/full-store behavior, repeated power cuts, reboot/watchdog/coredump. |
| OTA/security | Signed compatible images, secure boot, credential lifecycle, revocation and rollback/interruption. |
| Cloud/provider | TLS/broker ACL/OIDC, notification provider acceptance, app receipt and backup-caregiver drill. |
| Installed home | Four-node + hub placement, coverage, multi-day WAN/router outage, usability and false-positive/false-negative review. |

# 9. Git hygiene and release packaging

# from repository root
git status --short
git diff --cached --stat

# generated directories must remain untracked
build/
logs/
evidence/
node_modules/
test-results/
playwright-report/
__pycache__/
*.pyc

Keep code changes and documentation/progress changes separate when practical. The tested v3.4.2 source baseline was already pushed successfully to origin/main; later design-document updates can be committed as a dedicated documentation commit.

# 10. Handover checklist

Can explain node → hub → backend → PWA flow and identify each state owner.

Can explain why missing evidence is not a wellbeing conclusion and why battery cannot drive the care banner.

Can trace morning completion, missing morning, door-left-open, post-door inactivity, night-activity and battery high-drain from event to PWA.

Can edit a Device Schedule, identify config_version change and explain desired/applied semantics.

Can run make release-gate-final and interpret host vs physical acceptance.

Knows the current morning-bedroom restart behavior and does not alter it without explicit product/test update.

Knows which folders are generated and should not be committed.

Knows the remaining physical/HIL gates before any unattended pilot claim.