PARIVAR SATHI / GHAR SAJAG

# Ghar Sajag Hardware-Independent Low-Level Design

Current code flow, interfaces, state ownership, battery analytics and handoff gates

| Document version | 2.1 |
| --- | --- |
| Reference source | ParivarSathi_v1.5.4_PWA_BatteryAnalytics_v3_4_2 |
| Reference Git state | main / commit 0cc5ca6 (reported after push) |
| Date | 12 September 2026 |
| Status | v2.1 current implementation design; host release qualified, hardware/HIL pending |
| Audience | Firmware, backend, PWA, verification and integration engineers |

Boundary: this document describes the currently validated hardware-independent software. Real ESP32 GPIO/RF, power/charging, flash durability, signed OTA, provider delivery and installed-home acceptance remain separate physical/HIL gates.

# 1. Purpose, authority and delta from v2.0

This edition reconciles the design line with the implementation that was actually validated in ParivarSathi_v1.5.4_PWA_BatteryAnalytics_v3_4_2. It is the engineering handoff reference for the current host software. Where an older v2.0 description conflicts with this edition, this v2.1 implementation-status interpretation is authoritative.

| Area | Current v2.1 design status |
| --- | --- |
| Deterministic monitoring | Generic DomainEvent + RulesCore path retained; routine and activity rules consume versioned household configuration. |
| PWA | Backend-driven Parivar Sathi PWA at /; engineering lab at /lab. Browser-local state is presentation only. |
| Routine expansion | Configurable morning sequence, night bathroom/common-room counts, PIR visit merge, door-left-open threshold and post-door inactivity are implemented. |
| Battery analytics | NodeMessage v2 optional cumulative power telemetry + backend BatteryAnalyticsService + Device Health/Devices read model implemented. |
| Validation | 92 canonical connected scenarios and mandatory 22-test desktop/mobile Playwright release gate are current. |
| Physical product | Real GPIO/RF/power/flash/OTA/provider/installed-home acceptance remains pending. |

Key safety invariant — The software reports observed evidence, coverage and configured-rule outcomes. It does not infer that a resident is medically safe. Device/battery faults are operational health concerns and never independently turn the household-care banner red.

Figure 1. Current logical runtime architecture and dependency direction.

# 2. State ownership and dependency direction

Figure 2. Authoritative state boundaries.

| Owner | Owns | Must not own / infer |
| --- | --- | --- |
| Node | Sensor qualification, event identity, cumulative power counters, local retry/persist boundary | Care incidents, household UI state, family policy. |
| Hub | Authenticated ingest, local journal/outbox, trusted time/coverage, deterministic routine/activity state, applied config | Caregiver workflow ownership or battery calibration guesswork. |
| Backend | Tenant access, desired config, incident workflow, notification state, read models, battery history/analytics | Raw GPIO logic or UI-only truth. |
| PWA | Presentation and authorized commands; editor state only | Canonical event truth, missing-routine inference independent of backend, care alerts from battery-only state. |

Hub event/rule state changes remain deterministic and testable without GPIO/RF dependencies.

The backend read model is the source for care.alert, card details, device state and versioned settings presented by the PWA.

The Settings editor uses an explicit open-state guard so 2-second polling does not rebuild/close the editor while a user is editing.

Recent Important Events are ordered newest-first from backend chronology; rule-generated events retain their detection timestamp.

# 3. Canonical contracts and data structures

## 3.1 DomainEvent and NodeMessage v2

| Contract | Important fields / semantics |
| --- | --- |
| DomainEvent | EventKey(source_id, session_id, sequence), kind, location, monotonic_ms, occurred_at/received_at, uncertainty_s, battery_mv, sensor_type, RSSI, test flag. |
| NodeMessage v2 | Typed node identity + sequence, sensor/event/location, monotonic and optional wall-clock time, battery mV, RSSI, optional NodePowerTelemetry. |
| NodeAckMessage | Application acknowledgement class + exact event identity. Physical transport adapter may vary; identity/ACK semantics shall not. |
| NodePowerTelemetry | Cumulative deep_sleep, awake, sensor-active, radio TX/RX time; packet/retry, wake/heartbeat, boot/brownout counters. |

Node protocol policy
  wire schema = 2
  heartbeat = 60 s
  offline threshold = 3 missed heartbeats + 10 s grace
  retry delays = 200 / 600 / 1800 / 10000 ms (+ bounded jitter)

Validation rule — Power telemetry is rejected when sensor/radio active time exceeds total awake time. Battery voltage is bounded by the transport validator. Counter decreases are treated as reboot/reset boundaries by battery analytics.

## 3.2 Household ActivityRuleConfig

Figure 3. Household policy configuration path.

| Policy | Implemented configuration |
| --- | --- |
| Morning | enabled; start/end minute; completion window; bedroom/bathroom/kitchen locations. |
| Night | enabled; night window; bathroom/common locations; independent visit thresholds; visit merge interval. |
| Door | quiet-hours window; door-left-open timeout; post-door inactivity enable + timeout. |
| Daytime inactivity | enabled; active window; inactivity threshold. |
| Battery warning | battery_alert_percent is household configuration exposed under Device Schedules. |

Transport reliability constants (ACK/retry/heartbeat/offline timing) remain engineering-controlled and are intentionally not exposed as family-person settings.

# 4. Node runtime LLD

| Module | Current responsibility | Important current boundary |
| --- | --- | --- |
| N01 Node sensing | Qualifies PIR/reed/button input into typed DomainEvent values. Capability checks reject impossible event/sensor combinations in simulation. | Real GPIO levels, debounce, pull configuration and exact sensor electrical behavior pending. |
| N02 Node radio | Builds NodeMessage, preserves event key, retries according to bounded policy until application acknowledgement. | Actual ESP-NOW/BLE/Wi-Fi security/range/coexistence/RSSI behavior pending. |
| N03 Node power | Tracks battery mV and cumulative power-state counters needed by analytics. | Current-to-mAh coefficients are not measured by firmware; physical calibration is external. |
| N04 Node lifecycle | Versioned config/lifecycle seam and future OTA binding. | Secure boot, signed image verification, rollback and interrupted OTA pending. |
| N05 Node storage | Persist-before-retire model for queued events. | Flash partition, wear, CRC/commit marker and power-cut durability pending on target. |

# 5. Hub runtime LLD

| Module | Current responsibility | Key invariant / extension |
| --- | --- | --- |
| H01 Ingest | Validate/authenticate node messages, deduplicate identity, translate to DomainEvent. | Bad schema/capability is rejected before rules. |
| H02 Journal | Commit accepted events and maintain outbox semantics before durable acknowledgement. | Production flash adapter still required. |
| H03 Time | Supply trusted clock/local minute to pure rules. | Untrusted time suppresses time-bound absence/door decisions. |
| H04 Coverage | Track node/hub lease and observation eligibility independently from activity. | Unknown/fault coverage suppresses absence claims. |
| H05 Rules adapter | Own RoutineState/ActivityRuleState shell around S02 RulesCore. | Pure core remains hardware/network independent. |
| H06 Resident UI | Handle physical OK / Call Family / local resident interactions. | No user-facing arbitrary Privacy ON/OFF control. |
| H07 Lifecycle/config | Apply desired versioned household/node configuration. | Desired != applied until target confirmation in production. |
| H08 Cloud link | Publish committed outbox; maintain cloud freshness/ACK seam. | Host uses local bridge; MQTT/TLS provider integration pending. |
| H09 Interaction controller | Optional AI interaction lifecycle retained behind product profile. | Base P0 monitoring does not depend on AI/audio/provider. |

FreeRTOS target composition remains: radio_callback copies/authenticates into a bounded queue; one hub_event_task owns routine/coverage/applied-state mutation; cloud_sync_task drains committed outbox. Host execution calls the same logical components synchronously.

# 6. Deterministic routine/activity rule LLD

Figure 4. Morning sequence state machine as implemented in v3.4.2.

## 6.1 Morning sequence

A qualifying bedroom motion inside the configured morning window starts the sequence and records the start timestamp.

Bathroom and kitchen may occur in either order, but both must occur after the start and within morning_sequence_window_seconds.

Activity in bathroom/kitchen before bedroom does not satisfy the current sequence.

Completion creates MORNING_ROUTINE_COMPLETED as an expected event.

Pending completion is not the same as MISSING_MORNING_ACTIVITY; the top banner remains positive until a real concern is created.

Current implementation behavior: another bedroom motion while the sequence is active restarts the sequence and clears bathroom/kitchen flags. This is documented as a design-review item rather than silently treated as intended product behavior.

## 6.2 Night activity

Distinct visits are merged when repeated PIR events occur inside night_visit_merge_seconds.

A concern fires only when the count is greater than the configured threshold, not equal to it.

Bathroom and common-room counters are independent and reset when a daytime event proves the night window has ended.

Night activity concerns are care concerns: they highlight Night Activity and set the generic household banner to attention.

## 6.3 Door and inactivity

Door open during configured quiet hours can produce UnexpectedDoorOpen when mode/time are eligible.

DoorLeftOpen fires at the configured timeout only when HOME + covered + trusted time.

Door close after a long-open condition emits a resolving expected event with measured duration.

PostDoorInactivity starts from a door-close anchor and is cancelled by qualifying indoor motion before threshold.

Daytime inactivity uses last activity or the explicit monitoring-start anchor and is suppressed by Away/Paused/Visitor eligibility policy, unavailable coverage or untrusted time as defined by the connected simulator.

# 7. Backend LLD

| Module/service | Current implementation |
| --- | --- |
| B01 Identity/access | Membership/permission/expiry checks. Real OIDC token validation remains an adapter concern. |
| B02 Home/consent | Household, resident, mode/consent state and audit semantics. |
| B03 Ingest | Idempotent cloud event acceptance and backend receipt state. |
| B04 Incidents | Stable business keys; claim/acknowledge/resolve workflow. |
| B05 Notifications | Routing/scheduling/provider-result state; not a real push/SMS sender yet. |
| B06 Fleet/config | Versioned desired household configuration and device records. |
| B07 Read models/API | Caregiver/family projection, exact event labels, chronology, PWA HTTP bridge. |
| B08 Operations/data | Freshness, diagnostics, audit, persistence contract and operational history. |
| BatteryAnalyticsService / B12 trace code | SOC, energy rate, remaining runtime, confidence, high-drain and usage-pattern metrics from validated telemetry + calibration. |

The relational contract includes battery_history, battery_power_profiles and battery_usage_history in addition to core household/event/incident/configuration tables. The deterministic host still uses an in-memory runtime store; SQLite validates the schema contract and production PostgreSQL wiring remains pending.

# 8. Battery analytics LLD

Figure 5. Battery estimation path and calibration boundary.

| Stage | Algorithm / rule |
| --- | --- |
| Input validation | Reject invalid profile values; reject negative counters or sensor/radio time greater than awake time. |
| SOC | Piecewise non-linear 1S Li-ion curve from 3300 mV=0% through 4200 mV=100%; production curve/fuel-gauge can replace it. |
| Energy model | Base deep-sleep and awake states partition elapsed time; sensor/radio currents are incremental above awake base to avoid double counting. |
| Rate | Interval mAh converted to mAh/day. Median of recent valid rates reduces noise. |
| High drain | Recent-vs-baseline rate above calibrated high_drain_ratio marks HIGH and remaining-life prediction intentionally uses the higher recent rate. |
| Reboot/counter reset | Any cumulative counter decrease invalidates that delta interval; new boot segment may still yield LOW-confidence estimate. |
| Confidence | LOW until enough history; MEDIUM >=24h and >=3 samples; HIGH >=72h and >=8 samples. |
| Reserve | Usable remaining mAh protects configured reserve; low voltage does not over-promise remaining days. |

Physical acceptance dependency — Field prediction accuracy is not certified until deep-sleep, awake, sensor, TX/RX current, ADC/fuel-gauge accuracy, usable cell capacity, temperature/aging and UPS behavior are measured on the exact commercial hardware.

# 9. PWA and presentation LLD

| UI element | Authoritative behavior |
| --- | --- |
| Overall care banner | Generic only. Red: “Attention needed at home / Please check the highlighted routine below.” Positive: “Home looks normal / All is well at home.” Driven by backend care alert, not browser inference. |
| Morning Routine card | Completed / pending / missing are distinct. Missing highlights the card and care banner; pending does not. |
| I am OK card | Positive explicit check-in shown in green with relative age. Chronology remains visible alongside other events. |
| Main Door card | Shows open/closed, quiet-hours concern, long-open concern or post-door inactivity reason. |
| Night Activity card | Shows bathroom/common counts and configured-routine concern. |
| Device Health card | Battery/offline/high-drain operational concerns. Never changes household care banner by itself. |
| Settings → Device Schedules | Editable household-specific rule thresholds/locations and battery alert threshold; versioned backend state. |
| Validation panel | Engineering-only manual/run-all bridge for canonical scenarios; must be removed/hidden in production deployment. |

Dashboard card order in the validated PWA: Morning Routine → I am OK → Main Door → Night Activity → Device Health. Desktop and Android-sized mobile rendering are both release-gated.

# 10. End-to-end flows

## 10.1 Configurable morning completion

PIR motion → Node DomainEvent/NodeMessage
→ Hub authenticated ingest + journal
→ RulesCore.apply_activity_event
→ morning state update
→ when bathroom_seen && kitchen_seen: MORNING_ROUTINE_COMPLETED
→ backend timeline/read model
→ PWA Morning Routine = Completed; care banner remains positive

## 10.2 Missing morning concern

routine deadline/grace due + HOME + covered + trusted time
→ RulesCore/RoutineService absence decision
→ MISSING_MORNING_ACTIVITY incident
→ backend care.alert = true
→ PWA generic red banner + Morning Routine card holds specific reason

## 10.3 Battery high-drain concern

node cumulative power telemetry + battery mV
→ backend history + BatteryAnalyticsService
→ recent rate >> baseline
→ drain_status = HIGH + shorter estimated_days
→ Device Health / Devices highlighted
→ care.alert remains unchanged

## 10.4 Device schedule edit

Settings editor open
→ validated field values
→ backend versioned config patch
→ desired/applied simulation path
→ state refresh returns incremented config_version
→ editor remains stable during polling due scheduleEditorOpen guard

# 11. Validation and release design

Figure 6. Current release qualification stack.

| Layer | Current validated result |
| --- | --- |
| Requirement coverage | PASS; F01–F14 + E01–E10 mapped (24/24). |
| Contracts | PASS; 4 JSON schemas + examples/config/OpenAPI verification. |
| C++ | 94 checks PASS. |
| Python | 31 tests PASS. |
| JavaScript | 11 tests PASS. |
| Profiles/variants | Base + AI profiles PASS; four feature variants PASS. |
| Dummy streams | 5/5 PASS. |
| Canonical connected catalog | 92/92 PASS. |
| HTTP integration | 16/16 PASS. |
| Sanitizers / trace | AddressSanitizer/UBSan PASS; trace-enabled build/test PASS. |
| Legacy browser-e2e | PASS in latest native-WSL release-gate run. |
| Mandatory Playwright PWA | 22/22 PASS using one worker: 11 logical tests × desktop/mobile. |
| Final command | make release-gate-final → PASS on native WSL baseline. |

The canonical catalog is tests/functional/scenario_catalog.json and the generated manual plan is tests/MANUAL_FUNCTIONAL_VALIDATION.md. The PWA run-all path consumes the live HTTP results and the same assertion specification; Playwright verifies visible N/N PASS in a real Chromium DOM.

# 12. Known limitations and design-review items

| Item | Current status / required action |
| --- | --- |
| Morning bedroom restart | Implemented v3.4.2 behavior resets progress when another bedroom motion occurs. Product decision and dedicated regression case are recommended before hardware pilot. |
| Power accuracy | Algorithm exists; calibration is synthetic until measured on final board/sensor/regulator/cell. |
| Persistence | In-memory runtime + validated SQL schema; production flash/PostgreSQL adapters and power-cut evidence pending. |
| RF | Semantic transport tested; real ESP-NOW/BLE/Wi-Fi security/range/loss/coexistence pending. |
| OTA | Policy seams exist; signed firmware/secure boot/rollback interrupted-update evidence pending. |
| Notification delivery | Scheduler/state modeled; real push/SMS/provider acceptance and caregiver receipt drills pending. |
| Installed-home coverage | Sensor placement, dead zones, multi-resident interpretation and long-duration household pilot pending. |
| Performance/resource | Real ESP32 RAM/flash/CPU/current/thermal measurements pending. |

# 13. Engineering handoff gates

1.  Use the native WSL working tree as the official build/test/release environment on this laptop; /mnt/f is a backup/document/release-artifact location, not the qualification path.

2.  Before any code release candidate, run make release-gate-final and retain evidence only as generated artifacts, not as source-controlled build output.

3.  For a new behavior, update the canonical functional catalog or the relevant unit/integration suite before calling the feature release-complete.

4.  When real ESP32 hardware arrives, replace synthetic adapters incrementally without changing domain contracts/rules unless a measured constraint requires a reviewed design change.

5.  Do not claim unattended/pilot readiness until physical GPIO/RF/power/flash/OTA/provider/installed-home gates have recorded evidence.